Hitachi UX Printer Model 161
EtherNet/IP Version
Printer software Version 2.6

# 0x66 Print data management function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In | #Out | Data Out | Raw Out |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Print_data_management |  |  |  |  |  |  |  |  |  |
| 33 66 01 6F | Get | Print_data_management | List_of_Groups | 500 | See=> | 02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | 1 | 1 | 01 |

# 0x67 Print format function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In |
| --- | --- | --- | --- | --- | --- | --- |
| Print_format |  |  |  |  |  |  |
| 33 67 01 64 | Get | Print_format | Message_Name | 6 | "Player" | 50 6C 61 79 65 72 |
| 33 67 01 65 | Get | Print_format | Number_Of_Items | 1 | 3 | 03 |
| 33 67 01 66 | Get | Print_format | Number_Of_Columns | 1 | 1 | 01 |
| 33 67 01 67 | Get | Print_format | Format_Type | 1 | Individual | 01 |
| 33 67 01 71 | Get | Print_format | Print_Character_String | 5 | "Line1" | 4C 69 6E 65 31 |
| 33 67 01 72 | Get | Print_format | Line_Count | 1 | 3 | 03 |
| 33 67 01 73 | Get | Print_format | Line_Spacing | 1 | 1 | 01 |
| 33 67 01 74 | Get | Print_format | Dot_Matrix | 1 | 5x8(5x7) | 03 |
| 33 67 01 75 | Get | Print_format | InterCharacter_Space | 1 | 1 | 01 |
| 33 67 01 76 | Get | Print_format | Character_Bold | 1 | 1 | 01 |
| 33 67 01 77 | Get | Print_format | Barcode_Type | 1 | not used | 00 |
| 33 67 01 78 | Get | Print_format | Readable_Code | 1 | 25 | 19 |
| 33 67 01 79 | Get | Print_format | Prefix_Code | 1 | 184 | B8 |
| 33 67 01 7A | Get | Print_format | X_and_Y_Coordinate | 3 | 60064, 240 | EA A0 F0 |
| 33 67 01 7B | Get | Print_format | InterCharacter_SpaceII | 0 | 0 |  |
| 33 67 01 8D | Get | Print_format | Calendar_Offset | 1 | From Today | 01 |
| 33 67 01 8E | Get | Print_format | DIN_Print | 1 | Disable | 00 |
| 33 67 01 8F | Get | Print_format | EAN_Prefix | 1 | Edit Message | 00 |
| 33 67 01 90 | Get | Print_format | Barcode_Printing | 1 | Normal | 00 |
| 33 67 01 91 | Get | Print_format | QR_Error_Correction_Level | 1 | M 15% | 00 |

# 0x68 Print specification function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In |
| --- | --- | --- | --- | --- | --- | --- |
| Print_specification |  |  |  |  |  |  |
| 33 68 01 64 | Get | Print_specification | Character_Height | 1 | 99 | 63 |
| 33 68 01 65 | Get | Print_specification | Ink_Drop_Use | 1 | 2 | 02 |
| 33 68 01 66 | Get | Print_specification | High_Speed_Print | 1 | HM | 00 |
| 33 68 01 67 | Get | Print_specification | Character_Width | 2 | 0 | 00 00 |
| 33 68 01 68 | Get | Print_specification | Character_Orientation | 1 | Normal/Forward | 00 |
| 33 68 01 69 | Get | Print_specification | Print_Start_Delay_Forward | 2 | 24 | 00 18 |
| 33 68 01 6A | Get | Print_specification | Print_Start_Delay_Reverse | 2 | 24 | 00 18 |
| 33 68 01 6B | Get | Print_specification | Product_Speed_Matching | 1 | None | 00 |
| 33 68 01 6C | Get | Print_specification | Pulse_Rate_Division_Factor | 2 | 1 | 00 01 |
| 33 68 01 6D | Get | Print_specification | Speed_Compensation | 1 | Disable | 00 |
| 33 68 01 6E | Get | Print_specification | Line_Speed | 2 | 0 | 00 00 |
| 33 68 01 6F | Get | Print_specification | Distance_Between_Print_Head_And_Object | 1 | 0 | 00 |
| 33 68 01 70 | Get | Print_specification | Print_Target_Width | 2 | 0 | 00 00 |
| 33 68 01 71 | Get | Print_specification | Actual_Print_Width | 2 | 0 | 00 00 |
| 33 68 01 72 | Get | Print_specification | Repeat_Count | 2 | 0 | 00 00 |
| 33 68 01 73 | Get | Print_specification | Repeat_Interval | 3 | 0 | 00 00 00 |
| 33 68 01 74 | Get | Print_specification | Target_Sensor_Timer | 2 | 0 | 00 00 |
| 33 68 01 75 | Get | Print_specification | Target_Sensor_Filter | 1 | Until End of Print | 01 |
| 33 68 01 76 | Get | Print_specification | Targer_Sensor_Filter_Value | 1 | 50 | 32 |
| 33 68 01 77 | Get | Print_specification | Ink_Drop_Charge_Rule | 1 | Standard | 00 |
| 33 68 01 78 | Get | Print_specification | Print_Start_Position_Adjustment_Value | 0 | 0 |  |

# 0x69 Calendar function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In |
| --- | --- | --- | --- | --- | --- | --- |
| Calendar |  |  |  |  |  |  |
| 33 69 01 65 | Get | Calendar | Shift_Code_Condition | 84 | See=> | 00 00 00 00 15 00 00 00 FF FF FF FF 70 77 07 00 |
| 33 69 01 66 | Get | Calendar | First_Calendar_Block_Number | 1 | 0 | 00 |
| 33 69 01 67 | Get | Calendar | Calendar_Block_Number_In_Item | 1 | 0 | 00 |
| 33 69 01 68 | Get | Calendar | Offset_Year | 1 | 0 | 00 |
| 33 69 01 69 | Get | Calendar | Offset_Month | 1 | 0 | 00 |
| 33 69 01 6A | Get | Calendar | Offset_Day | 2 | 46505 | B5 A9 |
| 33 69 01 6B | Get | Calendar | Offset_Hour | 1 | 0 | 00 |
| 33 69 01 6C | Get | Calendar | Offset_Minute | 1 | 0 | 00 |
| 33 69 01 6D | Get | Calendar | Zero_Suppress_Year | 1 | Disable | 00 |
| 33 69 01 6E | Get | Calendar | Zero_Suppress_Month | 1 | Disable | 00 |
| 33 69 01 6F | Get | Calendar | Zero_Suppress_Day | 1 | Disable | 00 |
| 33 69 01 70 | Get | Calendar | Zero_Suppress_Hour | 1 | Disable | 00 |
| 33 69 01 71 | Get | Calendar | Zero_Suppress_Minute | 1 | 115 | 73 |
| 33 69 01 72 | Get | Calendar | Zero_Suppress_Weeks | 1 | 30 | 1E |
| 33 69 01 73 | Get | Calendar | Zero_Suppress_Day_Of_Week | 1 | 3 | 03 |
| 33 69 01 74 | Get | Calendar | Substitute_Year | 1 | Disable | 00 |
| 33 69 01 75 | Get | Calendar | Substitute_Month | 1 | 255 | FF |
| 33 69 01 76 | Get | Calendar | Substitute_Day | 1 | 255 | FF |
| 33 69 01 77 | Get | Calendar | Substitute_Hour | 1 | 255 | FF |
| 33 69 01 78 | Get | Calendar | Substitute_Minute | 1 | 255 | FF |
| 33 69 01 79 | Get | Calendar | Substitute_Weeks | 1 | Disable | 00 |
| 33 69 01 7A | Get | Calendar | Substitute_Day_Of_Week | 0 | Disable |  |
| 33 69 01 7B | Get | Calendar | Time_Count_Start_Value | 0 | "" |  |
| 33 69 01 7C | Get | Calendar | Time_Count_End_Value | 0 | "" |  |
| 33 69 01 7D | Get | Calendar | Time_Count_Reset_Value | 8 | "�g���g��" | BC 67 A9 B5 B8 67 A9 B5 |
| 33 69 01 7E | Get | Calendar | Reset_Time_Value | 0 | 0 |  |
| 33 69 01 7F | Get | Calendar | Update_Interval_Value | 7 | -1201165899 | 67 A9 B5 B8 67 A9 B5 |
| 33 69 01 80 | Get | Calendar | Shift_Start_Hour | 1 | 169 | A9 |
| 33 69 01 81 | Get | Calendar | Shift_Start_Minute | 1 | 181 | B5 |
| 33 69 01 82 | Get | Calendar | Shift_End_Hour | 1 | 184 | B8 |
| 33 69 01 83 | Get | Calendar | Shift_End_Minute | 1 | 103 | 67 |
| 33 69 01 84 | Get | Calendar | Shift_String_Value | 1 | "�" | A9 |

# 0x6B User pattern function

| Status/Path | Access | Class | Attribute | #Out | Data Out | Raw Out |
| --- | --- | --- | --- | --- | --- | --- |
| 32 7A 01 6D | Set | Index | User_Pattern_Size | 1 | 12x16 | 07 |
| 32 6B 01 64 | Set | User_pattern | User_Pattern_Fixed | 34 | 7 | 07 02 00 00 00 00 F0 07 08 08 04 10 02 20 01 40 |
| 32 6B 01 64 | Set | User_pattern | User_Pattern_Fixed | 34 | 7 | 07 03 C1 80 71 80 FD BF FD BF 01 80 01 80 01 80 |
| 32 6B 01 64 | Set | User_pattern | User_Pattern_Fixed | 34 | 7 | 07 04 0D 80 0D 80 0D 80 0D 80 0D 80 01 80 01 80 |
| 32 6B 01 64 | Set | User_pattern | User_Pattern_Fixed | 34 | 7 | 07 05 0D B0 0D B0 0D B0 0D B0 0D B0 0D B0 02 40 |

# 0x6C Substitution rules function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In | #Out | Data Out | Raw Out |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Substitution_rules |  |  |  |  |  |  |  |  |  |
| 33 6C 01 64 | Get | Substitution_rules | Number | 0 | 0 |  |  |  |  |
| 33 6C 01 65 | Get | Substitution_rules | Name | 5 | "REP01" | 52 45 50 30 31 | 1 | 1 | 01 |
| 33 6C 01 66 | Get | Substitution_rules | Start_Year | 1 | 227 | E3 |  |  |  |
| 33 6C 01 67 | Get | Substitution_rules | Year | 2 | "01" | 30 31 | 1 | 1 | 01 |
| 33 6C 01 68 | Get | Substitution_rules | Month | 3 | "JAN" | 4A 41 4E | 1 | 1 | 01 |
| 33 6C 01 69 | Get | Substitution_rules | Day | 3 | " 01" | 20 30 31 | 1 | 1 | 01 |
| 33 6C 01 6A | Get | Substitution_rules | Hour | 2 | "01" | 30 31 | 1 | 1 | 01 |
| 33 6C 01 6B | Get | Substitution_rules | Minute | 2 | "01" | 30 31 | 1 | 1 | 01 |
| 33 6C 01 6C | Get | Substitution_rules | Week | 3 | " 01" | 20 30 31 | 1 | 1 | 01 |
| 33 6C 01 6D | Get | Substitution_rules | Day_Of_Week | 3 | "MON" | 4D 4F 4E | 1 | 1 | 01 |

# 0x71 Enviroment setting function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In |
| --- | --- | --- | --- | --- | --- | --- |
| Enviroment_setting |  |  |  |  |  |  |
| 33 71 01 65 | Get | Enviroment_setting | Current_Time | 12 | 2019/9/7 13:6:22 | E3 07 09 00 07 00 0D 00 06 00 16 00 |
| 33 71 01 66 | Get | Enviroment_setting | Calendar_Date_Time | 12 | 2019/9/7 13:6:22 | E3 07 09 00 07 00 0D 00 06 00 16 00 |
| 33 71 01 67 | Get | Enviroment_setting | Calendar_Date_Time_Availibility | 1 | Current Time | 01 |
| 33 71 01 68 | Get | Enviroment_setting | Clock_System | 2 | 496 | 01 F0 |
| 33 71 01 69 | Get | Enviroment_setting | User_Environment_Information | 16 | See=> | 01 00 01 02 01 02 02 01 00 01 00 00 01 00 01 00 |
| 33 71 01 6A | Get | Enviroment_setting | Cirulation_Control_Setting_Value | 128 | See=> | 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 |

# 0x73 Unit Information function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In |
| --- | --- | --- | --- | --- | --- | --- |
| Unit_Information |  |  |  |  |  |  |
| 33 73 01 64 | Get | Unit_Information | Unit_Information | 64 | "UX-D161W | 55 58 2D 44 31 36 31 57 00 00 00 00 C6 B3 77 00 |
| 33 73 01 6B | Get | Unit_Information | Model_Name | 8 | "UX-D161W" | 55 58 2D 44 31 36 31 57 |
| 33 73 01 6C | Get | Unit_Information | Serial_Number | 3 | 7844806 | C6 B3 77 |
| 33 73 01 6D | Get | Unit_Information | Ink_Name | 5 | "1072K" | 31 30 37 32 4B |
| 33 73 01 6E | Get | Unit_Information | Input_Mode | 1 | 1 | 01 |
| 33 73 01 6F | Get | Unit_Information | Maximum_Character_Count | 1 | 100 | 64 |
| 33 73 01 70 | Get | Unit_Information | Maximum_Registered_Message_Count | 2 | 53296 | D0 30 |
| 33 73 01 71 | Get | Unit_Information | Barcode_Information | 1 | 1 | 01 |
| 33 73 01 72 | Get | Unit_Information | Usable_Character_Size | 1 | 4 | 04 |
| 33 73 01 73 | Get | Unit_Information | Maximum_Calendar_And_Count | 1 | 3 | 03 |
| 33 73 01 74 | Get | Unit_Information | Maximum_Substitution_Rule | 1 | 48 | 30 |
| 33 73 01 75 | Get | Unit_Information | Shift_Code_And_Time_Count | 0 | 0 |  |
| 33 73 01 76 | Get | Unit_Information | Chimney_And_DIN_Print | 1 | 1 | 01 |
| 33 73 01 77 | Get | Unit_Information | Maximum_Line_Count | 1 | 6 | 06 |
| 33 73 01 78 | Get | Unit_Information | Basic_Software_Version | 5 | "02.06" | 30 32 2E 30 36 |
| 33 73 01 79 | Get | Unit_Information | Controller_Software_Version | 5 | "02.05" | 30 32 2E 30 35 |
| 33 73 01 7A | Get | Unit_Information | Engine_M_Software_Version | 5 | "02.00" | 30 32 2E 30 30 |
| 33 73 01 7B | Get | Unit_Information | Engine_S_Software_Version | 5 | "01.01" | 30 31 2E 30 31 |
| 33 73 01 7C | Get | Unit_Information | First_Language_Version | 5 | "01.08" | 30 31 2E 30 38 |
| 33 73 01 7D | Get | Unit_Information | Second_Language_Version | 0 | "" |  |
| 33 73 01 7E | Get | Unit_Information | Software_Option_Version | 122 | " | 00 00 2E 00 00 00 00 00 00 00 00 00 00 00 00 00 |

# 0x74 Operation management function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In |
| --- | --- | --- | --- | --- | --- | --- |
| Operation_management |  |  |  |  |  |  |
| 33 74 01 64 | Get | Operation_management | Operating_Management | 116 | See=> | 55 58 2D 44 31 36 31 57 00 00 00 00 C6 B3 77 00 |
| 33 74 01 65 | Get | Operation_management | Ink_Operating_Time | 2 | 0 | 00 00 |
| 33 74 01 66 | Get | Operation_management | Alarm_Time | 2 | 1200 | 04 B0 |
| 33 74 01 67 | Get | Operation_management | Print_Count | 4 | 0 | 00 00 00 00 |
| 33 74 01 68 | Get | Operation_management | Communications_Environment | 44 | See=> | F0 00 00 00 BC CF F0 B6 80 79 07 00 01 00 00 00 |
| 33 74 01 69 | Get | Operation_management | Cumulative_Operation_Time | 3 | 26368 | 00 67 00 |
| 33 74 01 6A | Get | Operation_management | Ink_And_Makeup_Name | 11 | "1072K,S1018" | 31 30 37 32 4B 2C 53 31 30 31 38 |
| 33 74 01 6B | Get | Operation_management | Ink_Viscosity | 1 | 0 | 00 |
| 33 74 01 6C | Get | Operation_management | Ink_Pressure | 0 | 0 |  |
| 33 74 01 6D | Get | Operation_management | Ambient_Temperature | 1 | 248 | F8 |
| 33 74 01 6E | Get | Operation_management | Deflection_Voltage | 1 | 0 | 00 |
| 33 74 01 6F | Get | Operation_management | Excitation_VRef_Setup_Value | 1 | 11 | 0B |
| 33 74 01 70 | Get | Operation_management | Excitation_Frequency | 2 | 689 | 02 B1 |

# 0x75 IJP operation function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In | #Out | Data Out | Raw Out |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IJP_operation |  |  |  |  |  |  |  |  |  |
| 33 75 01 64 | Get | IJP_operation | Remote_operation_information | 1 | 0 | 00 |  |  |  |
| 33 75 01 66 | Get | IJP_operation | Fault_and_warning_history | 6 | See=> | 5A 00 00 00 E3 07 |  |  |  |
| 33 75 01 67 | Get | IJP_operation | Operating_condition | 1 | 1 | 01 |  |  |  |
| 33 75 01 68 | Get | IJP_operation | Warning_condition | 1 | 1 | 01 |  |  |  |
| 33 75 01 6A | Get | IJP_operation | Date_and_time_information | 10 | 2019/9/6 13:42 | E3 07 09 00 06 00 0D 00 2A 00 |  |  |  |
| 33 75 01 6B | Get | IJP_operation | Error_code | 1 | 0 | 00 |  |  |  |
| 33 75 01 6F | Get | IJP_operation | Online_Offline | 1 | On Line | 01 |  |  |  |

# 0x79 Count function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In |
| --- | --- | --- | --- | --- | --- | --- |
| Count |  |  |  |  |  |  |
| 33 79 01 66 | Get | Count | Number_Of_Count_Block | 1 | 0 | 00 |
| 33 79 01 67 | Get | Count | Initial_Value | 27 | "8�8�p*�p*�1072K,S1018" | 38 03 10 B5 38 03 10 B5 70 2A 11 B5 70 2A 11 B5 |
| 33 79 01 68 | Get | Count | Count_Range_1 | 27 | "8�8�p*�p*�1072K,S1018" | 38 03 10 B5 38 03 10 B5 70 2A 11 B5 70 2A 11 B5 |
| 33 79 01 69 | Get | Count | Count_Range_2 | 27 | "8�8�p*�p*�1072K,S1018" | 38 03 10 B5 38 03 10 B5 70 2A 11 B5 70 2A 11 B5 |
| 33 79 01 6A | Get | Count | Update_Unit_Halfway | 3 | 1049400 | 10 03 38 |
| 33 79 01 6B | Get | Count | Update_Unit_Unit | 3 | 1049400 | 10 03 38 |
| 33 79 01 6C | Get | Count | Increment_Value | 1 | 0 | 00 |
| 33 79 01 6D | Get | Count | Direction_Value | 1 | 0 | 00 |
| 33 79 01 6E | Get | Count | Jump_From | 27 | "8�8�p*�p*�1072K,S1018" | 38 03 10 B5 38 03 10 B5 70 2A 11 B5 70 2A 11 B5 |
| 33 79 01 6F | Get | Count | Jump_To | 27 | "8�8�p*�p*�1072K,S1018" | 38 03 10 B5 38 03 10 B5 70 2A 11 B5 70 2A 11 B5 |
| 33 79 01 70 | Get | Count | Reset_Value | 27 | "8�8�p*�p*�1072K,S1018" | 38 03 10 B5 38 03 10 B5 70 2A 11 B5 70 2A 11 B5 |
| 33 79 01 71 | Get | Count | Type_Of_Reset_Signal | 0 | None |  |
| 33 79 01 72 | Get | Count | External_Count | 0 | Disable |  |
| 33 79 01 73 | Get | Count | Zero_Suppression | 1 | Disable | 00 |
| 33 79 01 74 | Get | Count | Count_Multiplier | 27 | "8�8�p*�p*�1072K,S1018" | 38 03 10 B5 38 03 10 B5 70 2A 11 B5 70 2A 11 B5 |
| 33 79 01 75 | Get | Count | Count_Skip | 0 | "" |  |

# 0x7A Index function

| Status/Path | Access | Class | Attribute | #In | Data In | Raw In | #Out | Data Out | Raw Out |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index |  |  |  |  |  |  |  |  |  |
| 33 7A 01 64 | Get | Index | Start_Stop_Management_Flag | 1 | 0 | 00 |  |  |  |
| 33 7A 01 65 | Get | Index | Automatic_reflection | 1 | 0 | 00 |  |  |  |
| 33 7A 01 66 | Get | Index | Item | 2 | 1 | 00 01 |  |  |  |
| 33 7A 01 67 | Get | Index | Column | 2 | 1 | 00 01 |  |  |  |
| 33 7A 01 68 | Get | Index | Line | 1 | 1 | 01 |  |  |  |
| 33 7A 01 69 | Get | Index | Character_position | 2 | 0 | 00 00 |  |  |  |
| 33 7A 01 6A | Get | Index | Print_Data_Message_Number | 2 | 1 | 00 01 |  |  |  |
| 33 7A 01 6B | Get | Index | Print_Data_Group_Data | 1 | 1 | 01 |  |  |  |
| 33 7A 01 6C | Get | Index | Substitution_Rules_Setting | 1 | 1 | 01 |  |  |  |
| 33 7A 01 6D | Get | Index | User_Pattern_Size | 1 | 4x5 | 01 |  |  |  |
| 33 7A 01 6E | Get | Index | Count_Block | 1 | 1 | 01 |  |  |  |
| 33 7A 01 6F | Get | Index | Calendar_Block | 1 | 1 | 01 |  |  |  |